# Technical Assessment of Meteorological Fault Injection Fidelity, Detector Robustness, and Edge AI Architectures for Embedded Weather Station Networks

Automatic weather stations (AWS) deployed in operational meteorological networks require continuous data validation to ensure sensor readings remain accurate for numerical weather prediction, hydrometeorological modeling, and automated climate monitoring1. Operating these remote observation posts in harsh environmental conditions exposes sensors to mechanical wear, electronic drift, power instability, and severe weather events, leading to complex fault signatures1. Implementing machine learning (ML) and heuristic quality control pipelines directly on resource-constrained microcontrollers, such as the Espressif ESP32, enables real-time sensor validation, immediate data sanitization, and automated edge-level sensor quarantine5. This research report provides a rigorous technical evaluation of the fault injection mechanisms, feature engineering frameworks, and fusion logic within the SkyGuard AI system7. It addresses physical realism, algorithmic overfitting, mathematical fusion deadlocks, and introduces an edge-native TinyML architecture optimized for low-power microcontroller deployment8.

## Evaluation of Synthetic Fault Realism and Meteorological Physical Fidelity

Evaluating synthetic fault injection mechanisms requires comparing injected numerical mutations against the physical failure dynamics of field-deployed weather sensors1. Weather observation instrumentation relies on precision transducers—such as platinum resistance thermometers (PT100/PT1000) for ambient temperature, capacitive polymer sensors for relative humidity, and piezoresistive silicon elements for barometric pressure—connected through analog-to-digital converters (ADCs) to processing microcontrollers1. An analysis of the six synthetic fault routines reveals varying degrees of alignment with natural atmospheric physics and electronic failure modes1.

| Fault Type | Synthetic Injection Mechanics | Physical AWS Failure Mechanics | Realism Assessment |
| --- | --- | --- | --- |
| Spike | Single-reading jump ranging from to above or below the 48-hour rolling mean7. | Voltage surges, electrostatic discharge, or digital communication bit-flips yielding transient jumps of to 4. | Unrealistic Magnitude: deviations represent mathematically extreme anomalies that rarely occur without hardware clipping, creating an unnaturally simplistic classification task. |
| Frozen Value | Bit-exact replacement of values across 6 to 8 consecutive hourly timesteps without signal noise7. | Mechanical jamming of anemometer cups, ice accretion on sensor shields, or frozen ADC registers1. | Moderately Unrealistic: Physical sensors exhibit thermal drift and rail noise ( LSB jitter); bit-exact float repeating ignores standard analog-to-digital noise floors. |
| Drift | Strictly monotonic linear or polynomial ramp over 20 to 49 steps accumulating to offset7. | Sensor calibration decay, optical lens fogging, or slow chemical contamination of capacitive sensing elements3. | Unphysical Trajectory: Forcing monotonic drift across 20–49 hours completely overrides the natural diurnal solar cycle of atmospheric air temperature and humidity4. |
| Dropout | Single-reading assignment of missing values (NaN) across individual parameters7. | Telemetry packet loss over cellular/LoRa networks, bus read timeouts (I2C/SPI), or system power resets1. | Highly Realistic: Accurately reflects standard digital communications failures and telemetry buffer dropouts observed in field-deployed stations. |
| Multivariate Inconsistency | Simultaneous positive temperature shift () and positive relative humidity shift () with static pressure7. | Sensor cross-talk, short circuits from water ingress across PCB traces, or local heating events. | Thermodynamically Invalid: Heating ambient air exponentially increases saturation vapor pressure, naturally driving relative humidity down unless moisture is independently advected. |
| Sensor Fail-Low | Fixed boundary injection with minor Gaussian noise (, , ) across 4 steps7. | Physical ground shorts, cable cuts, or complete sensor element detachment pulling analog input pins to ground4. | Plausible Dynamics, Unphysical Values: A surface barometric pressure of is physically impossible on Earth; ground shorts trigger zero-scale ADC counts ( or ). |

### Thermodynamic and Physical Inconsistencies in Synthetic Generation

The mathematical mechanics of synthetic injection introduce critical departures from atmospheric physics7. In operational surface networks, atmospheric temperature () and relative humidity () are tightly coupled through the Clausius-Clapeyron relation, which governs saturation vapor pressure () as a function of temperature:

Relative humidity represents the ratio of actual vapor pressure () to saturation vapor pressure (), expressed as:

When ambient temperature increases rapidly without an influx of atmospheric moisture,  increases exponentially, causing relative humidity to drop4. The synthetic multivariate injector forces temperature up by  while simultaneously raising relative humidity by 7. This co-positive movement violates thermodynamic relationships, creating an unphysical signature4.

Furthermore, surface air temperature follows a diurnal forced cycle driven by solar irradiance (), modeled as a periodic function with a 24-hour frequency ():

The synthetic drift generator imposes a strictly monotonic polynomial function over 20 to 49 consecutive hours7:

Because  monotonically increases over spans exceeding a full diurnal cycle, it suppresses natural nighttime radiative cooling and daytime solar heating4. Real-world sensor calibration drift presents as a slow offset added to the underlying diurnal oscillation () rather than a monotonic trajectory that overwrites natural daily cycles3.

## Detector Robustness, Overfitting Analysis, and Decision Fusion Mathematics

Analyzing the detection module requires evaluating its heuristic rule engine, the unsupervised Isolation Forest model, and the decision fusion architecture that merges these two components7. Structural vulnerabilities in the rule structures and a mathematical deadlock in the fusion threshold constants impair system performance7.

### Decision Fusion Mathematics and System Deadlock

The decision fusion engine aggregates model outputs and heuristic evidence using a weighted linear combination7. The Isolation Forest output is squashed via a logistic transformation into a model percentage score (), while the rule engine provides a maximum rule confidence ()7. The combined anomaly score is defined as:

Where the operational weights are set to  and 7. System logic flags a reading as anomalous if any of three conditional bypasses evaluate to True:

- [cite: 7]

- [cite: 7]

- [cite: 7]

Evaluating these parameters mathematically reveals that the Isolation Forest model is completely isolated from determining anomaly verdicts7:

#### Scenario A: Primary Heuristic Rule Fires

When any standard rule fires (e.g., frozen_value, drift, confirmed_spike, sensor_fail_low, physical_bounds, or dropout), its rule confidence is set between  and 7. Because , condition 3 evaluates to True unconditionally, bypassing the fusion score7. The final decision is affirmative regardless of the model output7.

#### Scenario B: Weak Heuristic Rule Fires

When a single suspicious multivariate event (multivariate_single) fires, its rule confidence is assigned a value of 7. For condition 1 to evaluate to True, the required model score is derived as:

Because  is bounded by definition to , achieving a value of  is impossible. Consequently, a weak rule event can never trigger an anomaly flag, regardless of model confidence7.

#### Scenario C: No Heuristic Rule Fires (Model-Only Anomaly)

When no heuristic rules fire (), evaluating condition 1 yields:

This condition cannot be satisfied7. Furthermore, condition 2 requires , which is also impossible given the clipping function on 7.

#### Fusion Deadlock Conclusion

Under current configuration parameters, the anomaly flag decision is determined entirely by whether 7. The Isolation Forest model is mathematically incapable of raising an independent anomaly flag or corroborating weak heuristic signals7.

### Algorithmic Overfitting to Injected Mechanics

The detection heuristics are heavily specialized to the exact numerical parameters of the fault injectors, creating systemic vulnerabilities when exposed to natural sensor anomalies7:

- CUSUM Monotonic Direction Gate: The CUSUM drift engine requires 8 consecutive steps in the same direction (CUSUM_DIRECTION_STREAK_REQUIRED = 8) alongside an accumulator threshold ()7. This direction gate was added to match the strictly monotonic 20–49 step synthetic drift7. Natural sensor drift, however, fluctuates alongside ambient atmospheric turbulence and wind gusts3. A single step reversal caused by natural environmental noise resets the 8-step directional counter, preventing the CUSUM engine from flagging real-world drift3.

- Precision-Based Freeze Verification: The frozen value detector checks for rounded duplicate values over 6 consecutive timesteps (FROZEN_CONSECUTIVE_REQUIRED = 6)7. This relies directly on the synthetic injector's bit-exact duplicate float generator7. Operational AWS sensors experience ambient electromagnetic interference and ADC thermal noise ( noise floor)1. These low-level fluctuations break exact equality checks, causing real frozen sensors to go undetected7. Conversely, during stable winter temperature inversions, barometric pressure naturally remains static across a  reporting grid for several hours, triggering false positive detections4.

- Static Fail-Low Thresholds: The fail-low rule checks against static lower boundaries (, , )7. While this reliably catches the synthetic generator's unphysical values (), it fails to detect intermediate supply voltage drops (e.g., a power sag dropping a  reading to )4.

## Benchmark Labeled AWS Datasets for Supervised Anomaly Classification

Transitioning from overfitted synthetic heuristics to robust machine learning requires training on peer-reviewed, field-validated meteorological anomaly datasets12. Several public datasets provide high-resolution observations with standardized quality control flags suitable for supervised multi-class training14.

| Dataset Name | Temporal Resolution & Scale | Quality Control Flag Mechanics | Primary Sensor Anomaly Types | Supervised Training Utility |
| --- | --- | --- | --- | --- |
| NOAA GHCN-Daily (Global Historical Climatology Network)16 | Daily records across 100,000+ global surface stations16. | Standardized single-character quality flags (QFLAGs) appended to every observation18. | Spatial outliers (S), internal inconsistency (I), failed streak checks (K), gap checks (G), and physical bounds violations (X)4. | Excellent for training supervised classifiers on global operational sensor failure modes across diverse climate regimes4. |
| USCRN Quality Controlled Datasets (U.S. Climate Reference Network)15 | 5-minute and 1-hour intervals across 114 high-precision stations21. | Diagnostic flag vectors generated by triple-redundant sensor arrays21. | Individual sensor divergence, calibration drift, solar radiation shield fan failures, ground shorts, and precipitation gauge errors3. | Optimal for fine-grained multi-sensor consistency modeling and cross-sensor temperature/humidity coupling3. |
| SIGMA AWS Dataset (Snow Height and Ice Calculation for Greenland)14 | 1-hour resolution from extreme Arctic observational towers14. | Multi-level quality flags separating raw data (Level 1.0) from verified clean data (Level 1.1/2.0)23. | Extreme riming, optical window icing, persistent wind-induced freezing, radiant flux inconsistencies, and battery sags23. | Ideal for stress-testing models against harsh environmental conditions where natural extremes mimic hardware failures23. |

Utilizing these datasets enables training supervised multi-class models (such as Random Forests or Gradient Boosted Trees) on real sensor failure signatures, removing reliance on overfitted synthetic assumptions12.

## Edge AI Architecture and Optimization for ESP32 Deployment

Deploying real-time time-series fault detection pipelines onto resource-constrained microcontrollers—such as the Espressif ESP32 (XTensa LX6 dual-core, 240 MHz, 512 KB SRAM) or ESP32-S3 (XTensa LX7 with vector extensions)—requires optimizing model size, memory footprint, and execution latency5. Standard floating-point machine learning frameworks exceed available SRAM and introduce runtime overhead10. To achieve sub-millisecond processing, three lightweight Edge AI architectures offer distinct operational trade-offs8.

### Compiled Decision Tree Ensembles via emlearn

Instead of running heavy Python or C++ interpreters, tree-based models (Random Forest, Extra Trees) can be compiled directly into native C code using the emlearn framework9. emlearn converts trained decision trees into C structures or nested conditional execution branches9. This approach eliminates runtime heap allocations, recursive function calls, and external software dependencies9. Executing purely as native C code, emlearn models process inputs in sub-millisecond timeframes while using minimal SRAM28.

### Quantized 1D Convolutional Autoencoders via TFLite Micro / ESP-DL

For unsupervised temporal sequence analysis, a 1D Convolutional Autoencoder (1D-CAE) captures complex spatial and temporal correlations across multi-channel sensor arrays without requiring manually engineered heuristic rules8. The model accepts a sliding temporal window  (where  hourly timesteps and  channels: temperature, pressure, humidity), compresses the sequence into a low-dimensional representation, and reconstructs the clean sequence 33. The anomaly metric is computed as the Mean Squared Reconstruction Error (MSRE):

When  exceeds a calibrated threshold , the sequence is flagged as anomalous8. Converting the model to 8-bit integer precision (INT8) using post-training quantization allows it to run efficiently on the ESP32-S3 using vector instruction extensions (ESP-NN), enabling fast matrix operations10.

### FastGRNN (Fast Gated Recurrent Neural Network)

Standard Recurrent Neural Networks (RNNs) and Long Short-Term Memory (LSTM) networks maintain hidden states that effectively capture sequential dependencies, but their memory requirements often exceed microcontroller limits25. FastGRNN addresses this limitation by parameterizing recurrent matrices with low-rank and sparse structures26. The recurrent hidden state  is computed as:

To optimize memory, the weight matrices are decomposed using low-rank factorization and joint non-linear sparsity masks:

Where  is a diagonal matrix,  and  are low-rank matrices, and  is a binary sparsity matrix26. FastGRNN reduces memory overhead up to  compared to standard LSTMs while preserving accuracy on non-stationary meteorological time series26.

### Comparative Edge Performance Matrix on ESP32 Hardware

| Performance Metric | Compiled Native Trees (emlearn) | Quantized 1D-CAE (TFLite Micro / ESP-DL) | FastGRNN (EdgeML Framework) |
| --- | --- | --- | --- |
| Model Type | Supervised Compiled C Ensemble | Unsupervised Deep Reconstruction | Supervised Recurrent Network |
| Binary Size (Flash) | [cite: 28] | [cite: 10, 37] | [cite: 26, 36] |
| Runtime SRAM Footprint | [cite: 28] | [cite: 10, 38] | [cite: 26, 36] |
| Inference Latency (240 MHz) | [cite: 28] | [cite: 10, 38] | [cite: 26] |
| Hardware Acceleration | None required (Pure C branches)9 | ESP-NN SIMD Vector Instructions10 | Scalar Assembly Optimizations26 |
| Fault Coverage Capabilities | High for known static/temporal rules9 | Exceptional for novel structural shifts8 | High for sequential dynamic drift26 |
| Training Pipeline Complexity | Extremely Low (scikit-learn)9 | Moderate (TensorFlow/PyTorch)34 | High (Custom constrained Adam)26 |

## Breakthrough System Architecture and Edge Refactoring Roadmap

To build a high-performance fault detection system capable of running reliably on low-power edge hardware, the processing pipeline should be structured into a multi-tiered execution model5. This architecture prioritizes fast deterministic screening before invoking deeper machine learning models7.

Tier 1: Deterministic Physical Screening
  └─ Checks physical boundaries (-10C to +55C, 850-1080 hPa)
  └─ Identifies missing values (NaN / NULL)
  └─ Execution time: < 1 microsecond (Fails immediately raise Flag: 100%)
       │
       ▼ (Passes Tier 1)
Tier 2: Supervised Compiled Tree Inference (emlearn)
  └─ Evaluates 12 temporal features (ROC_1h, ROC_3h, Volatility Z-Score)
  └─ Evaluates physical coupling (Dew-point depression)
  └─ Execution time: < 0.1 milliseconds
       │
       ├─► [High Confidence Fault Detected] ──► Flag Anomaly & Update Health Tracker
       │
       ▼ [Uncertain / Borderline Verdict]
Tier 3: Quantized Temporal Reconstruction (1D-CAE / ESP-DL)
  └─ Evaluates 24-hour window reconstruction error (MSRE)
  └─ Validates complex temporal dynamics
  └─ Execution time: 3.5 - 8.0 milliseconds
       │
       ▼
Final Decision Fusion & Sensor Quarantine Verdict

### Tier 1: Deterministic Physical Screening

The input stream passes through a lightweight physical boundary filter7. Readings outside valid operational limits (, , ) or digital dropouts (NaN) are immediately flagged with  confidence7. This tier executes in , filtering gross sensor failures without consuming machine learning compute cycles7.

### Tier 2: Supervised Compiled Tree Inference (emlearn)

Data passing Tier 1 moves to a supervised Multi-Class Random Forest compiled to native C using emlearn9. The model evaluates 12 engineered features:

- Normalized 1-hour and 3-hour rates of change (, )7

- Rolling 24-hour volatility -scores7

- Dew-point depression ()

- ADC precision streak counts7

The compiled decision engine classifies the sample into specific fault categories (Spike, Drift, Noise, Frozen, Multivariate_Inconsistency, or Normal) in 7.

### Tier 3: Quantized Temporal Reconstruction (1D-CAE)

If Tier 2 yields an uncertain classification for subtle or novel anomalies, the sequence is passed to an INT8-quantized 1D Convolutional Autoencoder running via ESP-DL8. Processing a 24-hour window, the 1D-CAE evaluates overall sequence consistency33. If the Mean Squared Reconstruction Error exceeds the threshold (), the sample is flagged as anomalous8.

### Roadmap for Codebase Implementation

#### 1. Re-balance Fusion Constants

Update threshold constants in config.py to restore the model's contribution to anomaly decisions7:

- Lower FUSION_ANOMALY_THRESHOLD from  to 7.

- Lower MODEL_ALONE_OVERRIDE_THRESHOLD from  to 7.

- Reduce unconfirmed rule base confidences (multivariate_single )7.

- This setup allows high model confidence scores () to elevate weak heuristic signals above the decision threshold7.

#### 2. Upgrade the Anomaly Injection Module

Refactor data/anomaly_injector.py to align synthetic generation with atmospheric physics7:

- Adjust spike magnitudes to  with exponential decay recovery tails4.

- Replace bit-exact frozen floats with a stationary baseline that includes ADC thermal noise ( random walk)1.

- Superimpose drift ramps onto the station's diurnal solar cycle () rather than overwriting it4.

- Enforce saturation vapor pressure limits () during multivariate injections4.

#### 3. Implement Column-Aware Overlap Tracking

Update data/anomaly_injector.py:has_overlap() to track claimed time intervals per parameter dictionary (claimed_spans[col])7. This permits simultaneous single-sensor faults on separate channels (e.g., a frozen pressure sensor alongside a temperature spike) while maintaining overlap protection for multi-column fault events7.

#### 4. Optimize Memory Buffering for Embedded Systems

Replace dynamic array resizes with fixed ring buffers allocated in static SRAM (static float buffer[48][3])7. Pre-allocating memory prevents heap fragmentation and guarantees deterministic processing times during extended field deployments on ESP32 microcontrollers30.

#### Works cited

- MEASUREMENTS AT AUTOMATIC WEATHER STATIONS, https://www.hidmet.gov.rs/data/download/wmo%20no.8_eng.pdf

- Guide to Meteorological Instruments and Methods of Observation, https://www.weather.gov/media/epz/mesonet/CWOP-WMO8.pdf

- (PDF) Sensor and Electronic Biases/Errors in Air Temperature, https://www.researchgate.net/publication/249604712_Sensor_and_Electronic_BiasesErrors_in_Air_Temperature_Measurements_in_Common_Weather_Station_Networks

- Comprehensive Automated Quality Assurance of Daily Surface, https://www.ncei.noaa.gov/pub/data/ghcn/daily/papers/durre-menne-etal2010.pdf

- Tiny Machine Learning Environment: - IRIS UniGe, https://unige.iris.cineca.it/retrieve/a4bb0c5a-b5a3-4350-a3a8-0403ac5ae25c/phdunige_%204779788.pdf

- (PDF) Real-Time Anomaly Detection for Industrial Robotic Arms, https://www.researchgate.net/publication/391764497_Real-Time_Anomaly_Detection_for_Industrial_Robotic_Arms_Using_Edge_Computing

- SKYGUARD_DETECTION_TECHNICAL_REFERENCE.md

- TinyNS: Platform-Aware Neurosymbolic Auto Tiny Machine Learning, https://pmc.ncbi.nlm.nih.gov/articles/PMC11200268/

- emlearn documentation - Read the Docs, https://emlearn.readthedocs.io/en/latest/source/README.html

- FORGE: Forward-Only Test-Time Adaptation for Integer-Only Vision, https://arxiv.org/html/2609.01683v1

- Fusing ERA5-Land and SMAP L4 for an improved global soil, https://essd.copernicus.org/articles/18/1061/2026/

- AWMC: Abnormal-Weather Monitoring and Curation Service Based, https://www.mdpi.com/2076-3417/12/20/10444

- Quality assessment and control of urban environmental sensors, https://www.tandfonline.com/doi/full/10.1080/20964471.2026.2623720

- (PDF) Quality-controlled meteorological datasets from SIGMA, https://www.researchgate.net/publication/376019086_Quality-controlled_meteorological_datasets_from_SIGMA_automatic_weather_stations_in_northwest_Greenland_2012-2020

- Global soil moisture data derived through machine learning trained, https://pmc.ncbi.nlm.nih.gov/articles/PMC8275613/

- Global Historical Climatology Network (GHCN) - Drought.gov, https://www.drought.gov/data-maps-tools/global-historical-climatology-network-ghcn

- Global Historical Climatology Network - Daily (GHCN-Daily), Version 3, https://www.ncei.noaa.gov/access/metadata/landing-page/bin/iso?id=gov.noaa.ncdc:C00861;view=iso

- North American historical monthly spatial climate dataset, 1901–2016, https://pmc.ncbi.nlm.nih.gov/articles/PMC7683623/

- This is a non-peer-reviewed preprint submitted to EarthArXiv., https://eartharxiv.org/repository/object/11163/download/20308/

- Comprehensive Automated Quality Assurance of Daily Surface, https://www.researchgate.net/publication/237322480_Comprehensive_Automated_Quality_Assurance_of_Daily_Surface_Observations

- Methods to Calculate Precipitation from Weighing Bucket Gauges, https://www.researchgate.net/publication/276112175_Methods_to_Calculate_Precipitation_from_Weighing_Bucket_Gauges_with_Redundant_Depth_Measurements

- Installation of the USCRN soil-climate instrumentation was, https://www.researchgate.net/figure/Installation-of-the-USCRN-soil-climate-instrumentation-was-completed-for-the-contiguous_fig1_258797215

- Quality-controlled meteorological datasets from SIGMA automatic, https://essd.copernicus.org/articles/15/5207/2023/

- Drivers of variability of atmospheric carbon monoxide in Australia, https://ro.uow.edu.au/ndownloader/files/50382111/1

- Knowledge Distillation and Quantization-Aware Compression for, https://etasr.com/index.php/ETASR/article/view/19973

- Embedded Machine Learning Using Microcontrollers in Wearable, https://www.researchgate.net/publication/363605825_Embedded_Machine_Learning_Using_Microcontrollers_in_Wearable_and_Ambulatory_Systems_for_Health_and_Care_Applications_A_Review

- MbedML: A Machine Learning Project for Embedded Systems, http://sedici.unlp.edu.ar/bitstream/handle/10915/125142/Documento_completo.pdf?sequence=1

- emlearn: Machine Learning inference engine for Microcontrollers, https://www.researchgate.net/publication/351939316_emlearn_Machine_Learning_inference_engine_for_Microcontrollers_and_Embedded_Devices

- Milliwatt sized Machine Learning on microcontrollers with emlearn, https://archive.fosdem.org/2025/events/attachments/fosdem-2025-4524-milliwatt-sized-machine-learning-on-microcontrollers-with-emlearn/slides/238880/FOSDEM_20_zDTNPqa.pdf

- ESP32-S3 Fall Detection with TFLite Micro - ForestHub AI, https://www.foresthub.ai/resources/esp32-s3-fall-detection-tflite

- TinyML-CAM: 80 FPS image recognition in 1 kB RAM - ResearchGate, https://www.researchgate.net/publication/364529914_TinyML-CAM_80_FPS_image_recognition_in_1_kB_RAM

- Real-Time Wearable sEMG Onset Detection and Phase ... - PMC, https://pmc.ncbi.nlm.nih.gov/articles/PMC13431384/

- Evaluation of a Quantized 1D-CNN for Real-Time ECG Arrhythmia, https://www.researchgate.net/publication/405277070_Evaluation_of_a_Quantized_1D-CNN_for_Real-Time_ECG_Arrhythmia_Classification_on_a_Low-Power_Embedded_System

- TensorFlow Lite Micro on ESP32 (2026) - IoT Digital Twin PLM, https://iotdigitaltwinplm.com/edge-ai-tensorflow-lite-micro-esp32-implementation-guide/

- An Auto-Encoder Based TinyML Approach for Real-Time Anomaly, https://www.researchgate.net/publication/364182863_An_Auto-Encoder_Based_TinyML_Approach_for_Real-Time_Anomaly_Detection

- Machine Learning on Mainstream Microcontrollers - ResearchGate, https://www.researchgate.net/publication/341210665_Machine_Learning_on_Mainstream_Microcontrollers

- Design of a Lightweight Edge-AI System for Predictive Maintenance, https://www.mdpi.com/2076-3417/16/11/5287

- Real-Time Predictive Maintenance with SRU and LSTM on ESP32, https://www.researchgate.net/publication/399234217_Shrinking_AI_Real-Time_Predictive_Maintenance_with_SRU_and_LSTM_on_ESP32